FIRMWARE ENGINEER QUESTIONS

Many microcontrollers have an internal PLL that allows adjusting the microcontroller system clock.  Describe how a PLL works, and list the typical PLL registers that must be initialized during system start up.

- Write a short pseudocode illustrating how that would be accomplished.

A 24 volt DC motor attached to a lead screw moves a tool in and out of a fixture.  It is required to control the speed, direction and position of the tool.

- List at least two types of motor that can do this job

- List the advantages and disadvantages of each type

- Suggest ways to know when the tool is at the limits.

  - Describe software ways this can be controlled.

- Suggest ways to know the position of the tool.

  - What software techniques apply to the position?

- Suggest ways the motor speed be controlled?

  - What are the software issues with the various speed control options?

- What is a PID controller and how would it apply to this task?

  - Write a very simple software pseudocode to demonstrate how to perform the above.

  - Describe what registers will need to be initialized.

  - Include code to communicate with an Android device to setup speed, position, etc.

  - Include an alarm for stuck motor.

  - Would an RTOS be useful for this application?  How would you implement it?

SOFTWARE DOCUMENTATION AND MAINTENANCE

- What is your favorite microcontroller code software documentation method, and why do you prefer it over other documentation methods?

- What is Revision Control, and how do you prefer to implement it?

- Describe the open source software “SUBVERSION”

  - When might Subversion be “too much”?

- Describe the open source software Doxygen.

DEBUGGING MICROCONTROLLER CODE

- Name some typical debugging techniques and tools.

- What is Eclipse?  How is it used for code development and debugging?

- Modern microcontrollers include various debug methods in the controller hardware.  Describe some of them.

- What is Boundary Scan?

  - What hardware is required to use boundary scan?

  - What software is required to use boundary scan?

  - How does boundary scan enable serial debugging of multiple devices on the same boundary scan network?

  - What software issues are the most difficult when using boundary scan?

BLUETOOTH

- Name some communication protocols that connected computing devices prior to Bluetooth.

- Describe the function and advantages / disadvantages of Bluetooth over other connection methods.

- Describe the function of the Bluetooth protocol stack.

- A new product is designed to use a gyro  / accelerometer to measure  walking distance traveled. What software tasks must be done to connect the new device to a cell phone using Bluetooth?

- What is Device Discovery?

- Why was Bluetooth 4.0 (low energy) adopted?

- What is a Mesh Network?

  - Describe software issues involved with setting up a Mesh Network.

OTHER SERIAL CONNECTION PROTOCOLS

- Discus the RS232 standard

  - Describe the RS232 hardware interface.

  - How many devices can be networked?

  - What is a USART?   Discus the microcontroller software issues required to implement a USART using RS232.

- What is a CAN Bus?

  - Where was CAN first applied?

  - Why is CAN more capable than RS232?

  - What type of microcontroller hardware is required to network three devices over a CAN network?

  - Describe the CAN hardware driver.

  - A microcontroller is required to measure a tank level and temperature and intruder alarm, and to periodically send to another Master microcontroller over I2C.  Various situations require different actions; sound an alarm, send status to Master, turn on a red light.

    - Write a short pseudocode to illustrate initializing the controller registers.  Include an interrupt routine  using a CASE STATEMENT to direct an IRQ to the desired subroutine.

  - Describe how to set up registers in an Ethernet PHY.

    - Where might you find an Ethernet protocol stack for the microcontroller?

    - What is Unicast?

    - What is Broadcast?

    - What is Multicast?

    - What is Medium Access Control?